COMMON BIKE PROBLEMS AND THEIR SOLUTIONS:

- Issue: Bike Starts but No Acceleration:

| Field | Content |
| --- | --- |
| Symptoms | • Self-start works and engine turns ON |
|  | • Engine idles but vehicle doesn’t move when throttle given |
|  | • No increase in RPM or very slow pickup |
| User Level Checks | 1) Check if fuel is available |
|  | 2) See if throttle cable is loose |
|  | 3) Check rear wheel freely rotates |
|  | 4) Observe any burning smell |
| Probable Causes | Empty/blocked fuel line, stuck throttle cable, CVT belt slip, clutch worn out |
| Immediate Fix | Add fuel, check throttle grip movement, rest 10 minutes and retry |
| Do NOT Do | Do not over-accelerate continuously, do not ride forcefully |
| Mechanic Required When | Vehicle doesn’t move even after throttle, burning smell, belt noise |
| Clarifying Questions | Did engine start normally? Any noise from back wheel? Fuel available? |
| Prevention | Service CVT every 6000 km, avoid sudden high throttle |

- Issue: Bike Stopped Suddenly While Riding:

| Field | Content |
| --- | --- |
| Symptoms | • Vehicle stopped abruptly while moving • Engine turned OFF on its own • Self-start may or may not work after stop • No prior warning sometimes |
| User Level Checks | 1) Check fuel level immediately 2) Turn key OFF and ON once 3) Try kick start instead of self 4) Check if engine overheated 5) Look for any fuel leakage |
| Probable Causes | Fuel empty, clogged fuel filter, overheating, loose spark plug cap, electrical cutoff due to side stand sensor |
| Immediate Fix | Move vehicle to safe side, add fuel if empty, wait 10 minutes to cool, try kick start |
| Do NOT Do | Do not keep cranking self-start continuously, do not open engine parts on road |
| Mechanic Required When | Repeated sudden stop, strong petrol smell, overheating frequently |
| Clarifying Questions | Was fuel available? Any warning light? Happened after long ride? |
| Prevention | Maintain minimum fuel level, regular service of filter, check plug cap tightness |

- Issue: Bike Not Starting at All:

| Field | Content |
| --- | --- |
| Symptoms | • No response when self-start pressed • Engine not cranking • Only click sound or completely dead • Kick start also fails |
| User Level Checks | 1) Check if fuel is available 2) See if battery terminals are tight 3) Check side-stand is up 4) Observe if headlights turning ON |
| Probable Causes | Dead battery, empty fuel, loose battery terminal, faulty side-stand sensor, blown fuse |
| Immediate Fix | Tighten battery terminals, try kick start with choke, replace 10A fuse if blown |
| Do NOT Do | Do not keep pressing self for long time, do not push start on heavy traffic |
| Mechanic Required When | No power even after charging, burning smell, repeated fuse failure |
| Clarifying Questions | Are lights working? Any click sound? Fuel available? |
| Prevention | Start vehicle daily once, avoid long parking without battery charge |

- Horn Not Working:

| Field | Content |
| --- | --- |
| Symptoms | • Horn gives no sound when pressed • Sound is very weak or broken • Headlight becomes dim while pressing horn • Issue started after rain/wash |
| User Level Checks | 1) Turn key ON and press horn |
|  | 2) Check 10A fuse near battery |
|  | 3) Inspect loose coupler near handle |
|  | 4) See if battery terminals tight |
| Probable Causes | Weak battery, blown fuse, loose connector, water in switch |
| Immediate Fix | Tighten terminals, replace 10A fuse, dry switch area, charge battery |
| Do NOT Do | Do not use higher amp fuse, do not join wires directly |
| Mechanic Required When | Fuse blows repeatedly, burning smell, wiring cut |
| Clarifying Questions | Are headlights working? Did rain occur? Self-start weak? |
| Prevention | Avoid pressure wash on switch, clean terminals monthly |

- Indicators Not Working:

| Field | Content |
| --- | --- |
| Symptoms | • No blink, fast blink |
| User Level Checks | 1) Check fuse, bulb socket |
| Probable Causes | Blown bulb, relay fault |
| Immediate Fix | Replace bulb |
| Do NOT Do | No direct wiring |
| Mechanic Required When | Relay/wiring cut |
| Clarifying Questions | Both sides? Battery weak? |
| Prevention | Electrical check |

- Flat Tyre:

| Field | Content |
| --- | --- |
| Symptoms | • Vehicle pulling to one side, wobbling, tyre looks low |
| User Level Checks | 1) Inspect tyre |
|  | 2) Check for nail |
|  | 3) Try filling air |
| Probable Causes | Puncture, valve leak, old tyre crack |
| Immediate Fix | Fill air and ride slowly to shop |
| Do NOT Do | Do not ride long on flat, no overfill |
| Mechanic Required When | Large cut, sidewall damage |
| Clarifying Questions | Front or rear tyre? Visible nail? |
| Prevention | Check pressure weekly |

- Brake Issue:

| Field | Content |
| --- | --- |
| Symptoms | • Lever loose • poor braking • squeaking |
| User Level Checks | 1) Test at low speed |
|  | 2) Check fluid/cable |
| Probable Causes | Worn pad, low fluid, loose cable |
| Immediate Fix | Adjust cable, ride slow |
| Do NOT Do | Do not ride fast |
| Mechanic Required When | No brake effect, leakage |
| Clarifying Questions | Disc or drum? Oil leak? |
| Prevention | Service every 5k km |

- Headlight Issue:

| Field | Content |
| --- | --- |
| Symptoms | • Light OFF or dim, flickering |
| User Level Checks | 1) Check fuse, bulb, battery |
| Probable Causes | Blown bulb, weak battery |
| Immediate Fix | Replace bulb, tighten coupler |
| Do NOT Do | No high-watt bulb |
| Mechanic Required When | Wiring burnt |
| Clarifying Questions | High beam working? |
| Prevention | Avoid water spray |

- Unusual Noise:

| Field | Content |
| --- | --- |
| Symptoms | Rattling while accel |
| User Level Checks | 1) Identify side, check tyre |
| Probable Causes | CVT belt, bearing |
| Immediate Fix | Ride slow |
| Do NOT Do | Don’t open CVT |
| Mechanic Required When | Loud metal noise |
| Clarifying Questions | Noise at idle? |
| Prevention | Regular service |

Note:

This assistant provides probable causes and basic guidance only. For repeated or serious issues, consult a qualified mechanic.